# Sphero RVR + Raspberry Pi Setup Guide

This guide covers setting up a Raspberry Pi 3 to communicate with a Sphero RVR using the Sphero Python SDK.

# Prerequisites

- Raspberry Pi 3 (or compatible model)

- Sphero RVR with updated firmware (connect to Sphero EDU App first)

- Raspberry Pi OS installed (this guide tested on Bookworm)

- Jumper wires for UART connection

- SSH access or keyboard/monitor for the Pi

# Step 1: Enable SSH (Optional)

If you need remote access to your Pi:

sudo systemctl enable ssh

sudo systemctl start ssh

To find your IP address:

hostname -I

# Step 2: Enable the Serial Port

- Edit the boot configuration file:

sudo nano /boot/firmware/config.txt

- Add or verify these lines are present at the end of the file:

enable_uart=1

dtoverlay=miniuart-bt

The second line moves Bluetooth to the mini UART, freeing up the primary UART for the RVR connection.

- Save the file and reboot:

sudo reboot

# Step 3: Verify UART Configuration

- After reboot, verify the serial port exists:

ls -l /dev/serial*

You should see /dev/serial0 pointing to ttyAMA0.

# Step 4: Create Serial Port Symlink

The Sphero SDK expects /dev/ttyS0, but Raspberry Pi uses /dev/serial0. Create a symlink:

- Create the symlink:

sudo ln -s /dev/serial0 /dev/ttyS0

- To make it persist across reboots, create a udev rule:

sudo nano /etc/udev/rules.d/99-sphero-serial.rules

- Add this line:

KERNEL=="ttyAMA0", SYMLINK+="ttyS0"

- Reload udev rules:

sudo udevadm control --reload-rules

# Step 5: Install pyenv and Python 3.10

The Sphero SDK requires Python 3.10 or earlier (it uses deprecated features removed in Python 3.11+). Raspberry Pi OS Bookworm only has Python 3.13, so we need pyenv.

- Install build dependencies:

sudo apt install -y build-essential libssl-dev zlib1g-dev libbz2-dev \

libreadline-dev libsqlite3-dev curl libncursesw5-dev xz-utils \

tk-dev libxml2-dev libxmlsec1-dev libffi-dev liblzma-dev

- Install pyenv:

curl https://pyenv.run | bash

- Add pyenv to your shell configuration:

echo 'export PYENV_ROOT="$HOME/.pyenv"' >> ~/.bashrc

echo 'export PATH="$PYENV_ROOT/bin:$PATH"' >> ~/.bashrc

echo 'eval "$(pyenv init -)"' >> ~/.bashrc

source ~/.bashrc

# Step 6: Install Python 3.10

This step compiles Python from source and takes 20-30 minutes on a Pi 3.

- Install Python 3.10:

pyenv install 3.10

- Set it as the global default:

pyenv global 3.10

- Verify:

python --version

Should show Python 3.10.x

# Step 7: Create Virtual Environment and Install SDK

- Create a virtual environment:

python -m venv ~/rvr-env

source ~/rvr-env/bin/activate

- Clone the SDK repository:

cd ~

git clone https://github.com/sphero-inc/sphero-sdk-raspberrypi-python.git

- Install the SDK and dependencies:

cd ~/sphero-sdk-raspberrypi-python

pip install -e .

pip install aiohttp

# Step 8: Make Settings Persist Across Reboots

Add the virtual environment activation to your shell configuration so it loads automatically on login:

echo 'source ~/rvr-env/bin/activate' >> ~/.bashrc

Verify your .bashrc has all the necessary lines:

tail -10 ~/.bashrc

You should see the pyenv configuration lines and the rvr-env activation line.

# Step 9: Wire the Raspberry Pi to the RVR

Connect the Raspberry Pi GPIO pins to the RVR's UART port on the expansion bay:

- Pi GND → RVR GND

- Pi TX (GPIO 14) → RVR RX

- Pi RX (GPIO 15) → RVR TX

# Step 10: Test the Connection

- Power on the RVR.

- Run an LED test script:

cd ~/sphero-sdk-raspberrypi-python/getting_started/observer/leds

python set_all_leds.py

The RVR's LEDs should change color, confirming the connection is working.

# Troubleshooting

## No module named 'sphero_sdk'

Make sure you're in the virtual environment (you should see (rvr-env) in your prompt) and that the SDK is installed:

source ~/rvr-env/bin/activate

cd ~/sphero-sdk-raspberrypi-python && pip install -e .

## Serial port not found

Verify the serial port exists and the symlink is correct:

ls -l /dev/serial* /dev/ttyS0

## Permission denied on serial port

Add your user to the dialout group:

sudo usermod -a -G dialout $USER

Then log out and back in.